# 235X Memory Performance Tests

# Configuration

These tests were run on the Waveshare Core2350B board with PSRAM and Flash.  These tests were not meant to be exhaustive and are designed to show relative performance (to 1 cycle SRAM).  No work has been done to tune performance of PSRAM.  The core was running at stock 150Mhz and PSRAM was configured:

Max Select: 18, Min Deselect: 2, clock divider: 2

# Results ( tl;dr )

For Sequential Reads compared to SRAM, Flash was 26.07x slower, PSRAM cached was 18.74x slower, PSRAM no-cache was 27x slower.

For Random Reads compared to SRAM, Flash was 99.98x slower, PSRAM cached was 56.27x slower, PSRAM no-cache was 96.95x slower.

For Sequential Writes compared to SRAM, PSRAM cached was 60.75x slower, PSRAM no-cache was 27x slower.

For Random Writes compared to SRAM, PSRAM cached was 100.59x slower, PSRAM no-cache was 96.7x slower.

# Data

| Iteration Count | 327,680,000 | clock_hz | 150,000,000 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Name | Memory | Size | Msec | Cycles per Iteration | Overhead | Ratio : 1 |
| SEQ SRAM READ | 0x20002BC8 | 16384 | 9284829 | 4.25 | 3.25 |  |
| SEQ ROM READ | 0x10006158 | 16384 | 64040290 | 29.32 |  | 26.07 |
| SEQ PSRAM READ | 0x11000000 | 16384 | 48045599 | 21.99 |  | 18.74 |
| SEQ PSRAM NOCACHE READ | 0x14000000 | 16384 | 66090753 | 30.25 |  | 27.00 |
| RND SRAM READ | 0x20002BC8 | 16384 | 17476828 | 8.00 | 7.00 |  |
| RND ROM READ | 0x10006158 | 16384 | 233707964 | 106.98 |  | 99.98 |
| RND PSRAM READ | 0x11000000 | 16384 | 138220924 | 63.27 |  | 56.27 |
| RND PSRAM NOCACHE READ | 0x14000000 | 16384 | 227077953 | 103.95 |  | 96.95 |
| SEQ SRAM WRITE | 0x20002BC8 | 16384 | 13654031 | 6.25 | 5.25 |  |
| SEQ PSRAM WRITE | 0x11000000 | 16384 | 144189214 | 66.00 |  | 60.75 |
| SEQ PSRAM NOCACHE WRITE | 0x14000000 | 16384 | 70443211 | 32.25 |  | 27.00 |
| RND SRAM WRITE | 0x20002BC8 | 16384 | 18023094 | 8.25 | 7.25 |  |
| RND PSRAM WRITE | 0x11000000 | 16384 | 235572917 | 107.84 |  | 100.59 |
| RND PSRAM NOCACHE WRITE | 0x14000000 | 16384 | 227078144 | 103.95 |  | 96.70 |

# Code

https://github.com/FarLeftLane

# Analysis

TBD…